# Managed Switch

## ⚙️ What is a Managed Switch?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

A Managed Switch is an advanced type of network switch that gives full control over your network. Unlike unmanaged switches, which are plug-and-play, managed switches allow you to configure, monitor, and manage network traffic to improve performance, security, and reliability.

Used mainly in business, enterprise, or data center environments.

## 🔧 Key Features of a Managed Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Feature | Description |
| --- | --- |
| VLAN Support | Create Virtual LANs to segment traffic and improve security. |
| Port Management | Enable/disable ports, set speeds, or assign roles (e.g., trunk or access). |
| QoS (Quality of Service) | Prioritize critical traffic like VoIP, video, or business apps. |
| Monitoring & Logging | View real-time stats, traffic usage, and receive alerts. |
| Security Features | Control access via MAC filtering, port security, or 802.1X authentication. |
| Remote Management | Manage via web GUI, CLI (Command-Line Interface), or SNMP. |
| Redundancy Protocols | Supports Spanning Tree Protocol (STP) and others for loop prevention. |
| Link Aggregation | Combine multiple ports for higher bandwidth and redundancy. |

## 🏢 Common Use Cases

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

- Segmenting departments in a business using VLANs

- Prioritizing video calls and VoIP traffic using QoS

- Securing access to network resources

- Monitoring bandwidth usage and traffic patterns

- Creating failover links and redundancy in large networks

## 🖥️ Example Network Setup

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

[Internet]

↓

[Router]

↓

┌────────────┐

│Managed Switch│

└┬───┬───┬────┬┘

↓   ↓   ↓    ↓

VLAN 1  VLAN 2  VLAN 3

(HR)   (IT)   (Guests)

## 🆚 Managed vs Unmanaged Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Feature | Managed Switch | Unmanaged Switch |
| --- | --- | --- |
| Setup Required | Yes | No |
| VLAN Support | ✅ Yes | ❌ No |
| Remote Management | ✅ Web/CLI/SNMP | ❌ None |
| Security Control | ✅ MAC filtering, 802.1X | ❌ None |
| Monitoring | ✅ Real-time traffic stats | ❌ Not available |
| Price | Higher | Lower |
| Use Case | Business, enterprise | Home, small offices |

## 🎯 Common Brands and Models

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Brand | Example Managed Switches |
| --- | --- |
| Cisco | Catalyst series (e.g., 2960, 9200) |
| TP-Link | JetStream series |
| Netgear | ProSAFE managed switches |
| Ubiquiti | UniFi Switch series |
| MikroTik | CRS (Cloud Router Switch) |