# I-frame

## 🧱 What Is I-frame?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

I-frames in HDLC (High-Level Data Link Control) are a fundamental concept in synchronous and asynchronous data link layer communication. Let’s break it down step by step.

## HDLC Frame Types

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

HDLC frames are used to reliably transmit data over a point-to-point or multipoint link. There are three main types of HDLC frames:

- I-frames (Information frames) – carry user data and control information.

- S-frames (Supervisory frames) – used for flow and error control.

- U-frames (Unnumbered frames) – manage link control (setup, disconnect, mode changes).

## What I-frames Do

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

- Carry actual user data (payload) from upper layers.

- Include sequencing information for error detection and reliable delivery.

- Support flow control and acknowledgment through sequence numbers.

## Structure of an I-frame

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

An I-frame typically consists of:

- Address field – identifies the destination station.

- Control field – contains:

  - N(S): send sequence number (number of the current frame)

  - N(R): receive sequence number (acknowledgment number)

  - P/F bit: poll/final bit for command/response control

- Information field (Data) – the payload from upper layers

- Frame Check Sequence (FCS) – error-detection code

## Example of Control Field in I-frame

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

- N(S) = 5 → This is the 6th frame sent (sequence starts at 0).

- N(R) = 3 → The sender has successfully received frames up to 3 from the other side.

This sequencing ensures that frames are delivered in order and without loss.

## Key Features

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

- Reliable delivery: lost frames can be retransmitted.

- In-order delivery: sequence numbers maintain correct order.

- Flow control: prevents sender from overwhelming the receiver.

## Comparison

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

| Frame Type | Purpose |
| --- | --- |
| I-frame | Carries user data, supports sequencing & acknowledgment |
| S-frame | Supervisory only, no user data, manages flow/error control |
| U-frame | Unnumbered, manages link control (setup/disconnect) |

## In Simple Terms

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

I-frames in HDLC are the “data-carrying trucks” that also report back which trucks were received and in what order, making sure nothing gets lost on the way.