# 32-bit ASN

## 🌐 What Is 32-bit ASN?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

A 32-bit ASN (Autonomous System Number) is a unique 32-bit identifier used in BGP (Border Gateway Protocol) to identify an Autonomous System (AS) — a network or group of networks under a single routing policy.

It expands the ASN range beyond the original 16-bit limit to allow for over 4 billion unique ASNs.

## 🧠 Simple Definition:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

A 32-bit ASN is a larger version of the traditional ASN, used to uniquely identify a network in global BGP routing.
 It supports values from 0 to 4,294,967,295.

## 🚀 Why 32-bit ASNs Were Introduced

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

Originally, ASNs were 16-bit, limited to 65,536 values — too few for the modern internet.

- Introduced by RFC 4893

- Fully supported by most modern routers

- Solves ASN exhaustion

## 📊 ASN Numbering Overview

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

| Type | Value Range | Notes |
| --- | --- | --- |
| 16-bit ASN | 0 – 65,535 | Traditional ASN format |
| 32-bit ASN | 65,536 – 4,294,967,295 | Extended format |

Total range for 32-bit ASN = 4,294,967,296 possible ASNs

## 🧾 ASN Format

### 🔸 Decimal Format:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

AS131072

### 🔹 Dotted (ASDOT) Format:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

2.0 → (2 × 65536) + 0 = 131072

Format: high-order. Low-order

| Example | Decimal |
| --- | --- |
| 0.1 | ASN 1 |
| 1.10 | ASN 65546 |
| 2.100 | ASN 131236 |

## ✅ Advantages of 32-bit ASN

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

| Benefit | Explanation |
| --- | --- |
| 🆙 More ASNs available | Supports over 4 billion ASNs |
| 🌐 Supports future growth | Ensures every organization can have a unique ASN |
| ✅ Fully compatible | Supported by modern routers and BGP software |
| 🛡️ Used in IPv6 deployment | Often seen in newer networks and ISPs |

## ⚠️ Compatibility Notes

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

Older devices and software may not recognize 32-bit ASNs if not updated.
 Today, 32-bit ASN support is standard in most BGP implementations (Cisco, Juniper, Mikrotik, etc.).

## ✅ Summary

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [1/16/2026]

| Feature | Value |
| --- | --- |
| Name | 32-bit Autonomous System Number |
| Range | 0 – 4,294,967,295 |
| Introduced In | RFC 4893 (2007) |
| Format | Decimal or ASDOT (e.g., 1.10) |
| Purpose | Identify a network for BGP routing |
| Still in use? | Yes – with growing adoption |