# S-frames

## 🧱 What Is S-frames?

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

S-frames in HDLC (High-Level Data Link Control) are Supervisory frames that play a key role in flow and error control but do not carry user data. They are used to manage the reliable transmission of I-frames.

## HDLC Frame Types (Context)

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

HDLC frames are divided into three types:

- I-frames (Information frames) – carry user data + control info

- S-frames (Supervisory frames) – flow and error control (no user data)

- U-frames (Unnumbered frames) – link management (setup, disconnect, mode changes)

## What S-frames Do

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

- Acknowledge received I-frames

- Request retransmission of missing or erroneous I-frames

- Control the flow to prevent the sender from overwhelming the receiver

S-frames never carry actual user data, only control information.

## Types of S-frames

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| S-frame Type | Purpose |
| --- | --- |
| RR (Receive Ready) | Acknowledges received frames; ready to receive more |
| RNR (Receive Not Ready) | Tells sender to pause sending (receiver is busy) |
| REJ (Reject) | Requests retransmission of a specific frame due to error |
| SREJ (Selective Reject, optional) | Requests retransmission of a specific frame while accepting others |

## Structure of an S-frame

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

An S-frame includes:

- Address field – identifies the destination station

- Control field – contains:

  - Type of S-frame (RR, RNR, REJ, SREJ)

  - N(R): receive sequence number (acknowledgment)

  - P/F bit: poll/final bit for command/response control

- No information field (no user data)

- Frame Check Sequence (FCS) – error detection

## Example Flow

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

- Sender transmits I-frames 0–3

- Receiver receives frames 0–2, misses frame 3

- Receiver sends REJ with N(R)=3 → request retransmission of frame 3

- Sender retransmits frame 3

This ensures reliable and ordered delivery.

## Key Points

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

- S-frames are lightweight supervisory messages

- They control data flow and handle errors without carrying data

- Essential for reliable communication in HDLC and similar protocols (like LAPB in X.25)

## In Simple Terms

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

S-frames are the “traffic controllers” of HDLC: they tell the sender which frames were received, which need to be resent, and whether to slow down or pause sending.