# HDLC Frame

## 🧱 What Is an HDLC Frame?

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

HDLC (High-Level Data Link Control) is a Layer 2 (Data Link Layer) protocol used to transmit data reliably over point-to-point and multipoint links. It is a bit-oriented synchronous protocol developed by ISO and commonly used in WANs and serial interfaces (e.g., routers).

An HDLC frame is the standard format used by HDLC to encapsulate data — ensuring proper delivery, sequencing, and error detection.

## 🧠 Simple Definition:

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

An HDLC frame is a structured data unit used to encapsulate packets for transmission over synchronous serial links, ensuring reliable communication between two or more devices.

## 🧾 HDLC Frame Format

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Flag | Address | Control | Payload | FCS | Flag |

## 🔍 HDLC Frame Field Breakdown

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Field | Size | Description |
| --- | --- | --- |
| Flag | 1 byte | 0x7E – Marks the start/end of the frame |
| Address | 1 byte | Address of the receiving station (in PPP, usually 0xFF) |
| Control | 1 byte | Type of frame (I-frame, S-frame, U-frame) |
| Payload | Variable | The encapsulated data (like an IP packet) |
| FCS | 2 or 4 bytes | CRC for error checking |
| Flag | 1 byte | 0x7E – End of the frame |

## 📂 Types of HDLC Frames

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Frame Type | Control Field | Description |
| --- | --- | --- |
| I-frame (Information) | Sequence numbers | Carries user data |
| S-frame (Supervisory) | ACK, flow control | No data, just control |
| U-frame (Unnumbered) | Command & setup | Used for connection management |

## 🧠 Visual Example – HDLC Frame

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

Flag:              7E

Address:           FF

Control:           00            ; I-frame (info frame)

Payload:

45 00 00 3C 1C 46 40 00 40 06 B1 E6 C0 A8 01 64

5D B8 D8 22 04 D2 00 50 71 10 FA 3E 00 00 00 00

80 02 20 00 91 7C 00 00 02 04 05 B4 01 03 03 08

01 01 04 02

FCS (CRC):         3A 5B

Flag:              7E

## 🆚 HDLC vs PPP

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Feature | HDLC | PPP |
| --- | --- | --- |
| Addressing | Optional or fixed | Optional (0xFF) |
| Framing | Bit-oriented | Byte-oriented |
| Multiprotocol | Not native | Supports IP, IPv6, LCP, etc. |
| Flexibility | Basic framing | Extensible with options |

## ✅ Summary

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Feature | Value |
| --- | --- |
| Layer | OSI Layer 2 |
| Use case | WAN links, leased lines, serial links |
| Framing | Uses 0x7E as both start and end |
| Error check | CRC-based FCS |
| Frame types | I-frame (data), S-frame (ACK/control), U-frame (setup/commands) |