# Ethernet (Twisted Pair)

## 🧵 What Are Ethernet (Twisted Pair) Cables?

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

Ethernet twisted pair cables are the most common cables used in local area networks (LANs) to transmit data between devices such as computers, switches, and routers. They use pairs of copper wires twisted together to reduce electromagnetic interference (EMI).

## Ethernet (Twisted Pair) photo

# Source: Windows Screenshots (using Lightshot Program) at [12/18/2025]

## 📦 Belongs to:

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

✅ Types of Network Cables

Specifically under wired (copper-based) transmission media.

## 🧠 Key Features

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

| Feature | Description |
| --- | --- |
| Medium | Copper wires twisted in pairs |
| Signal Type | Electrical (digital) signals |
| Shielding | Can be shielded (STP) or unshielded (UTP) |
| Connector | RJ-45 (standard Ethernet plug) |
| Use | Most common for LAN, internet, VoIP, PoE |

## 🧾 Common Ethernet Cable Categories

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

| Category | Max Speed | Max Length | Typical Use |
| --- | --- | --- | --- |
| Cat5 | 100 Mbps (Fast Ethernet) | 100 m | Obsolete – used in older networks |
| Cat5e | 1 Gbps (Gigabit) | 100 m | Most common for home/office LAN |
| Cat6 | 1 Gbps (up to 55m), 10 Gbps (short distances) | 100 m | Used in high-speed modern networks |
| Cat6a | 10 Gbps | 100 m | Shielded, better for noise-heavy areas |
| Cat7 | 10 Gbps | 100 m | Fully shielded, supports higher bandwidth |
| Cat8 | 25–40 Gbps | 30 m | Data centers, short high-speed runs |

## 🧰 STP vs UTP

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

| Type | Description |
| --- | --- |
| UTP (Unshielded Twisted Pair) | Common in most homes/offices; cheaper, easier to install |
| STP (Shielded Twisted Pair) | Has extra shielding to protect from EMI; used in industrial or noisy environments |

## ✅ Advantages

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

- 💸 Inexpensive and widely available

- ⚡ Fast for typical LAN use (1–10 Gbps)

- 🔌 Simple plug-and-play with RJ-45 connectors

- 🔧 Easy to terminate, extend, or replace

## ❌ Disadvantages

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

- 📏 Limited distance (usually 100 meters)

- ⚡ More interference than fiber in noisy environments

- 🌧️ Susceptible to damage from weather/moisture (if outdoor and unshielded)

## 📌 Summary

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

| Attribute | Value |
| --- | --- |
| Cable Type | Ethernet (Twisted Pair) |
| Medium | Copper wire (UTP or STP) |
| Common Connectors | RJ-45 |
| Used In | LAN, Internet, VoIP, PoE |
| Popular Categories | Cat5e, Cat6, Cat6a, Cat7, Cat8 |