# Managed PoE Switch

## ⚙️ What is a Managed PoE Switch?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

A Managed PoE Switch is a network switch that provides both:

- ✅ Power over Ethernet (PoE) — to deliver electricity and data over the same Ethernet cable to compatible devices (like IP cameras, VoIP phones, access points),

- ✅ Advanced network control and configuration — via a management interface (web, CLI, SNMP).

This makes it ideal for enterprise, campus, and professional-grade networks that need power delivery and fine-tuned control over traffic, security, and performance.

## 🔧 Key Features of a Managed PoE Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Feature | Description |
| --- | --- |
| PoE Power Delivery | Powers devices like IP cameras, phones, and access points via Ethernet cables (802.3af/at/bt). |
| VLAN Support | Segment your network for security and organization (e.g., isolate IoT, guest, or voice traffic). |
| Port Control | Enable/disable ports, adjust speeds, and control power output per port. |
| QoS (Quality of Service) | Prioritize bandwidth for voice, video, or mission-critical apps. |
| Traffic Monitoring | See live usage, errors, or dropped packets on each port. |
| Security Settings | Use port security, MAC filtering, or 802.1X authentication to control access. |
| Remote Management | Configure and monitor the switch through web GUI, CLI (SSH/telnet), SNMP, or cloud. |
| Redundancy Protocols | Use STP, RSTP, or LACP to avoid loops and add link failover. |
| PoE Budget Management | Control and monitor how much total power is being used or allocated across ports. |

## 🏢 Example Use Case: Business Surveillance Setup

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

[Firewall / Router]

↓

[Managed PoE Switch]

┌───┬───┬───┬─────┬─────┐

↓   ↓   ↓   ↓     ↓

[VoIP] [AP] [IP Cam] [IP Cam] [Server]

VLAN2 VLAN1 VLAN3   VLAN3   VLAN0

- VLANs isolate voice, wireless, and surveillance traffic

- QoS ensures VoIP traffic gets top priority

- The admin can monitor port usage and remotely reboot a camera

## 🆚 Managed vs Unmanaged PoE Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Feature | Managed PoE Switch | Unmanaged PoE Switch |
| --- | --- | --- |
| Configuration | ✅ Web, CLI, SNMP | ❌ None (plug-and-play) |
| VLAN Support | ✅ Yes | ❌ No |
| Power Control | ✅ Set max watts per port | ❌ Always on if PoE is detected |
| Monitoring | ✅ Traffic, errors, PoE load | ❌ Not available |
| Security | ✅ MAC filtering, 802.1X | ❌ None |
| Redundancy | ✅ Supports STP, LACP, etc. | ❌ No |
| Cost | 💲💲 Higher | 💲 Lower |
| Use Case | Business, campus, smart IoT | Home, small office, basic CCTV |

## 🔌 Power Standards Supported

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Standard | Power Per Port | Typical Use |
| --- | --- | --- |
| 802.3af (PoE) | 15.4W | Phones, basic APs |
| 802.3at (PoE+) | 30W | IP cameras, dual-band APs |
| 802.3bt (PoE++) | 60W–100W | PTZ cameras, LED lighting, industrial IoT |

## 📦 Common Vendors & Models

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Vendor | Managed PoE Switch Series |
| --- | --- |
| Cisco | Catalyst 2960-X, 9200 |
| Ubiquiti | UniFi Switch (USW) |
| TP-Link | JetStream |
| Netgear | ProSAFE Smart Switches |
| MikroTik | CRS Series |
| Aruba | Instant On PoE Switches |