# Active PoE Switch

## ⚙️ What is an Active PoE Switch?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

An Active PoE Switch is a network switch that provides Power over Ethernet (PoE) using IEEE standard protocols like 802.3af, 802.3at (PoE+), or 802.3bt (PoE++). It is designed to automatically detect and safely power compatible devices (like IP cameras, VoIP phones, or wireless access points) over the same Ethernet cable used for data.

## 🔌 How Active PoE Works

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

- The switch checks whether a connected device supports PoE.

- If it does, the switch delivers regulated power based on the device’s needs.

- If not, it sends only data — no risk of damage to the device.

✅ This intelligent detection is what makes it "active".

## 🔧 Key Features of an Active PoE Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Feature | Description |
| --- | --- |
| Standard-Compliant | Follows IEEE 802.3af / 802.3at / 802.3bt standards. |
| Device Detection | Automatically checks if a device is PoE-compatible before sending power. |
| Smart Power Delivery | Supplies only the power required — safely and efficiently. |
| Safe for Any Device | Will not damage non-PoE devices (they get data only). |
| Flexible Deployment | Can power a mix of IP cameras, phones, access points, IoT devices, etc. |

## 🧪 PoE Standards

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Standard | Power per Port | Total Power Budget | Common Uses |
| --- | --- | --- | --- |
| 802.3af (PoE) | Up to 15.4W | ~100W total | Phones, cameras, basic APs |
| 802.3at (PoE+) | Up to 30W | ~200W+ | Dual-band APs, PTZ cameras |
| 802.3bt (PoE++) | 60W – 100W+ | Up to 720W+ | PoE lighting, smart displays, laptops |

## 🏢 Example Use Case: Office Setup with Active PoE

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

[Firewall/Router]

↓

[Active PoE Switch]

┌────┬────┬────┬────┐

↓    ↓    ↓    ↓

[AP] [IP Phone] [Camera] [Laptop]

✅     ✅        ✅       ❌

(Powered via PoE)     (Data only)

- Devices that support PoE are powered.

- Non-PoE devices (like laptops) get only data — safely.

## 🆚 Active PoE vs Passive PoE

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

| Feature | Active PoE | Passive PoE |
| --- | --- | --- |
| Standard | ✅ IEEE 802.3af/at/bt | ❌ Non-standard |
| Detection | ✅ Detects before powering | ❌ Always sends voltage |
| Safety | ✅ Safe for any device | ❌ Risk of damage to non-PoE |
| Voltage Control | ✅ Smart & regulated | ❌ Fixed voltage |
| Cost | 💲💲 Medium–High | 💲 Lower |
| Recommended For | Business, secure setups | Specialized or DIY projects |

## ✅ Benefits of Active PoE Switches

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [12/18/2025]

- ✔ Safe to use with any device

- ✔ Easy to deploy in mixed environments

- ✔ Future-proof and scalable

- ✔ Compatible with standard PoE devices from any vendor