# U-frames

## 🧱 What Is U-frames?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

U-frames in HDLC (High-Level Data Link Control) are Unnumbered frames that are primarily used for link control and management. Unlike I-frames (which carry user data) or S-frames (which handle flow/error control), U-frames are mostly used to establish, maintain, and terminate the connection.

## HDLC Frame Types (Quick Context)

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Frame Type | Purpose |
| --- | --- |
| I-frame (Information) | Carries user data, supports sequencing & acknowledgment |
| S-frame (Supervisory) | Controls flow & error, no user data |
| U-frame (Unnumbered) | Link setup, disconnect, mode control, special commands |

## What U-frames Do

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

- Set up and terminate a connection

- Change link modes (normal, asynchronous balanced mode, etc.)

- Perform link tests or diagnostics

- Support special control functions

U-frames can operate independently of sequence numbers, which is why they are called “unnumbered.”

## Common U-frame Commands

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| U-frame Command | Purpose |
| --- | --- |
| SABM (Set Asynchronous Balanced Mode) | Initiates a connection in balanced mode |
| DISC (Disconnect) | Terminates a connection |
| UA (Unnumbered Acknowledgment) | Acknowledges a U-frame command |
| FRMR (Frame Reject) | Indicates a protocol or frame error |
| SIM / RIM (Set/Reset Initialization Mode) | Initialize or reset the link |

## Structure of a U-frame

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

- Address field – destination station

- Control field – indicates the type of U-frame and command

- Optional Information field – rarely used

- Frame Check Sequence (FCS) – error detection

Unlike I-frames and S-frames, U-frames are flexible and lightweight, designed for management rather than data transport.

## Key Points

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

- U-frames do not require sequence numbers

- Mostly used for link establishment, termination, and control

- Can operate independently of data flow

## In Simple Terms

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

U-frames are the “command and control messages” of HDLC: they set up connections, tear them down, and handle special link functions, without carrying user data.