# Frame Relay

## 🧱 What Is Frame Relay?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Frame Relay is a WAN (Wide Area Network) protocol that operates at OSI Layer 2 (Data Link Layer). It is used to transmit data between geographically distant LANs over virtual circuits provided by a telecommunications carrier.

Originally designed for efficient and fast packet-switching, Frame Relay became popular in the 1990s for connecting branch offices and enterprise sites before technologies like MPLS, VPN, and broadband became common.

## 🧠 Simple Definition:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Frame Relay is a fast, low-overhead WAN protocol that sends variable-length frames over a virtual circuit between two locations — ideal for interconnecting remote LANs over long distances.

## 🔧 How Frame Relay Works

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

- Uses virtual circuits:

  - PVC (Permanent Virtual Circuit) – always available

  - SVC (Switched Virtual Circuit) – temporary

- Transmits data as frames — each frame contains addressing and control info to reach the correct destination.

- Connection-oriented but doesn’t guarantee delivery (no error correction).

## 📦 Frame Relay Frame Format

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Flag | Address (DLCI) | Control | Payload | FCS | Flag |

## 🔍 Frame Field Breakdown

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Field | Size | Description |
| --- | --- | --- |
| Flag | 1 byte (0x7E) | Marks start/end of frame |
| Address (DLCI) | 2 bytes or more | Data Link Connection Identifier (virtual circuit number) |
| Control | 1 byte | Typically 0x03 (UI frame) |
| Payload | Variable | Actual user data (e.g. IP packet) |
| FCS | 2 bytes | Frame Check Sequence (CRC) |
| Flag | 1 byte (0x7E) | End of frame |

## 🧠 Visual Example – Frame Relay Frame (Simplified)

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

Flag:             7E

Address (DLCI):   03 E0        ; DLCI 100, C/R=0

Control:          03           ; Unnumbered Information (UI)

Payload:

45 00 00 3C 1C 46 40 00 40 06 B1 E6 C0 A8 01 64

5D B8 D8 22 04 D2 00 50 71 10 FA 3E 00 00 00 00

80 02 20 00 91 7C 00 00 02 04 05 B4 01 03 03 08

FCS (CRC):        3A 5B

Flag:             7E

## ✅ Key Characteristics

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Feature | Description |
| --- | --- |
| Layer | Data Link Layer (Layer 2) |
| Addressing | Uses DLCI (Data Link Connection Identifier) |
| Encapsulation | Sends Layer 3 packets (e.g. IP) in frames |
| Error Control | No retransmission — relies on upper layers |
| Bandwidth | Shared but configurable (CIR – Committed Info Rate) |
| Topology | Point-to-point or multipoint |

## 🆚 Frame Relay vs Other WAN Techs

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Feature | Frame Relay | PPP | MPLS |
| --- | --- | --- | --- |
| Connection Type | Virtual Circuit | Physical Link | Label-switched |
| Multiprotocol | Yes | Yes | Yes |
| Error Recovery | No | Yes | Yes (optional) |
| Speed | Medium | Medium | High |

## 🔄 Typical Use Case

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

- Connecting remote branch offices to a central data center

- Carried over leased lines or telco networks

- Often replaced today by VPN, SD-WAN, or MPLS

## 🧾 Summary

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [ChatGPT] on [1/16/2026]

| Term | Value |
| --- | --- |
| Protocol Type | WAN, Layer 2 |
| Frame Format | Flag, DLCI, Control, Payload, CRC, Flag |
| Use Cases | Long-distance LAN interconnection |
| Modern Status | Legacy / mostly replaced |