# Passive PoE Switch

## ⚠️ What is a Passive PoE Switch?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

A Passive PoE Switch is a type of Power over Ethernet switch that supplies a constant, unregulated voltage over Ethernet cables — without negotiating with connected devices. Unlike active PoE switches, it does not follow IEEE standards (like 802.3af/at/bt) and always sends power, regardless of whether the connected device needs it or not.

## 🔌 How Passive PoE Works

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

- It sends a fixed DC voltage (commonly 24V or 48V) directly over specific Ethernet pins.

- It does not detect if the device supports PoE — it just powers it anyway.

- This makes it cheaper, but riskier if used improperly.

## ⚙️ Key Characteristics

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Feature | Description |
| --- | --- |
| No Handshake | No negotiation with the device — power is always sent. |
| Fixed Voltage | Supplies a set voltage (e.g., 24V or 48V) continuously. |
| Non-Standard | Does not conform to IEEE PoE standards like 802.3af/at. |
| Lower Cost | Typically cheaper than active PoE switches. |
| Simpler Design | Often used in specialized or proprietary setups (like MikroTik or Ubiquiti). |

## ⚠️ Important Warning

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

Passive PoE can damage non-PoE devices if plugged in, because it does not check compatibility. Always ensure:

- The device is rated for the voltage used (e.g., 24V Passive PoE).

- You’re using compatible equipment (usually from the same vendor).

## 🧱 Use Cases for Passive PoE Switches

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Where Used | Why |
| --- | --- |
| Small wireless setups | For powering wireless bridges or access points |
| Rural ISP installations | To run long Ethernet + power cables to antennas |
| Proprietary systems | Often designed for specific hardware (e.g., MikroTik RouterBOARDs) |

## 🆚 Passive PoE vs Active (802.3af/at/bt) PoE

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Feature | Passive PoE | Active PoE (Standard) |
| --- | --- | --- |
| Power Negotiation | ❌ None (always on) | ✅ Yes (safe detection) |
| Standards | ❌ Non-standard | ✅ IEEE 802.3af/at/bt |
| Voltage | Fixed (e.g. 24V or 48V) | Automatically adjusts (up to 100W) |
| Device Safety | ❌ Risk of damage if mismatched | ✅ Safe for any device |
| Cost | 💲 Lower | 💲💲 Higher |
| Best For | Specialized or matched systems | General and enterprise use |

## ✅ When to Use a Passive PoE Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

- You are using all PoE-compatible devices with known voltage requirements

- You want lower cost in a controlled environment

- You are working with vendor-specific solutions (e.g., Ubiquiti AirMax or MikroTik gear)