# Bridge

## 🌉 What is a Bridge in Networking?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

A Bridge is a Layer 2 (Data Link Layer) network device that is used to connect two or more network segments and make them function as a single network. It filters and forwards traffic based on MAC addresses, reducing collisions and managing traffic intelligently.

✅ Think of it as a device that "bridges" two LANs together.

## 🔧 What a Bridge Does

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Task | Description |
| --- | --- |
| Connects Networks | Links two separate LAN segments to work as one. |
| Filters Traffic | Forwards data only when needed (based on MAC addresses). |
| Reduces Collisions | Segments the network to reduce traffic and improve efficiency. |
| Learns MAC Addresses | Keeps a table of MAC addresses to know where to send frames. |

## 📶 Basic Diagram

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

[LAN Segment A]     [LAN Segment B]

[PC1]---+             +---[PC3]

|             |

[ Bridge ]------[PC2]

- Devices on both sides can communicate.

- The bridge forwards frames only when the destination is on the other segment.

## 🧱 Where Bridges Are Used

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

- To segment traffic in large Ethernet networks

- To extend networks across floors or buildings

- In older networks, before switches became common

## 🆚 Bridge vs Switch

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Feature | Bridge | Switch |
| --- | --- | --- |
| Ports | Typically 2–4 | 4–48 or more |
| Performance | Slower (software-based in old models) | Faster (hardware-based) |
| MAC Table | Learns and stores MAC addresses | Same (but more scalable) |
| Use Today | Mostly obsolete | Standard in modern networks |

## 🧠 Types of Bridges

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Type | Description |
| --- | --- |
| Local Bridge | Connects two LANs on the same physical location |
| Remote Bridge | Connects LANs over a WAN (using VPN or leased lines) |
| Wireless Bridge | Connects two network segments over Wi-Fi (e.g., point-to-point link) |

## ✅ Benefits

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

- ✔ Reduces unnecessary traffic

- ✔ Creates a more efficient LAN

- ✔ Easy to configure in small setups

## ❌ Limitations

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

- ❌ Not suitable for large, complex networks

- ❌ Replaced by switches in modern infrastructure

- ❌ Limited port capacity

## 📌 Summary

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Term | Meaning |
| --- | --- |
| Bridge | Connects two or more LAN segments at Layer 2 using MAC addresses. |
| Purpose | Reduce traffic, manage data flow, and extend LANs. |
| Modern Equivalent | A switch (which is essentially a multi-port bridge). |