# Ethernet (Twisted Pair)

## 🧵 What Are Ethernet (Twisted Pair) Cables?

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

Ethernet twisted pair cables are the most common cables used in local area networks (LANs) to transmit data between devices such as computers, switches, and routers. They use pairs of copper wires twisted together to reduce electromagnetic interference (EMI).

## Ethernet (Twisted Pair) photo

# Source: ScreenShots Source: Provided by: [Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> Tool: [Lightshot Program] -> From [Site: Google Search: Images: Ethernet (Twisted Pair)] on [12/18/2025]

## 📦 Belongs to:

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

✅ Types of Network Cables

Specifically under wired (copper-based) transmission media.

## 🧠 Key Features

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Feature | Description |
| --- | --- |
| Medium | Copper wires twisted in pairs |
| Signal Type | Electrical (digital) signals |
| Shielding | Can be shielded (STP) or unshielded (UTP) |
| Connector | RJ-45 (standard Ethernet plug) |
| Use | Most common for LAN, internet, VoIP, PoE |

## 🧾 Common Ethernet Cable Categories

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Category | Max Speed | Max Length | Typical Use |
| --- | --- | --- | --- |
| Cat5 | 100 Mbps (Fast Ethernet) | 100 m | Obsolete – used in older networks |
| Cat5e | 1 Gbps (Gigabit) | 100 m | Most common for home/office LAN |
| Cat6 | 1 Gbps (up to 55m), 10 Gbps (short distances) | 100 m | Used in high-speed modern networks |
| Cat6a | 10 Gbps | 100 m | Shielded, better for noise-heavy areas |
| Cat7 | 10 Gbps | 100 m | Fully shielded, supports higher bandwidth |
| Cat8 | 25–40 Gbps | 30 m | Data centers, short high-speed runs |

## 🧰 STP vs UTP

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Type | Description |
| --- | --- |
| UTP (Unshielded Twisted Pair) | Common in most homes/offices; cheaper, easier to install |
| STP (Shielded Twisted Pair) | Has extra shielding to protect from EMI; used in industrial or noisy environments |

## ✅ Advantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

- 💸 Inexpensive and widely available

- ⚡ Fast for typical LAN use (1–10 Gbps)

- 🔌 Simple plug-and-play with RJ-45 connectors

- 🔧 Easy to terminate, extend, or replace

## ❌ Disadvantages

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

- 📏 Limited distance (usually 100 meters)

- ⚡ More interference than fiber in noisy environments

- 🌧️ Susceptible to damage from weather/moisture (if outdoor and unshielded)

## 📌 Summary

# Source: AI Source: Provided by: [Data Path: Persons Table -> ID Column: 1, Name_En Column: Motasem Essam] -> From: [Chatgpt (GPT‑5 Mini)] on [12/18/2025]

| Attribute | Value |
| --- | --- |
| Cable Type | Ethernet (Twisted Pair) |
| Medium | Copper wire (UTP or STP) |
| Common Connectors | RJ-45 |
| Used In | LAN, Internet, VoIP, PoE |
| Popular Categories | Cat5e, Cat6, Cat6a, Cat7, Cat8 |