# NAT Gateway

## 🌐 What is a NAT Gateway?

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

A NAT Gateway (Network Address Translation Gateway) is a network device or cloud service that enables private (internal) devices to access the internet while keeping their private IP addresses hidden. It translates internal IP addresses to a public IP address for outbound traffic — but blocks inbound connections unless explicitly allowed.

✅ In simple terms: A NAT Gateway lets devices in a private network go out to the internet, but prevents the internet from coming in uninvited.

## 🔧 What It Does

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

| Direction | Action |
| --- | --- |
| Outbound traffic | Rewrites private IPs to a public IP so internal devices can browse the web, download updates, etc. |
| Inbound traffic | Blocks unsolicited traffic from the internet (unless a rule exists for it) |

## 🏠 Simple Home Example

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

[PC: 192.168.1.100] ⇄ [NAT Gateway (Router)] ⇄ 🌐 Internet

↑

Public IP: 203.0.113.5

- Your PC sends a request to google.com → NAT Gateway changes the source IP to 203.0.113.5

- Google replies → NAT Gateway translates it back to 192.168.1.100

## ☁️ Example in AWS (Cloud)

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

In AWS, a NAT Gateway allows:

- Instances in private subnets to access the internet (for software updates, etc.)

- Without assigning them public IPs

- Inbound internet connections are not allowed unless initiated by the instance

## 🧠 NAT Gateway vs Internet Gateway

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

| Feature | NAT Gateway | Internet Gateway |
| --- | --- | --- |
| IP Translation | Yes (private → public, outbound) | No translation (routes traffic) |
| Inbound Access | ❌ Blocks unsolicited inbound | ✅ Allows inbound (with rules) |
| Use Case | Outbound access for private subnets | Internet access for public subnets |
| Public IP Required | Yes (for NAT Gateway) | Yes (for public instances) |

## ✅ Advantages

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

- ✅ Keeps internal devices secure and private

- ✅ Allows outbound internet access without public IPs

- ✅ Simplifies cloud network design (e.g., AWS, Azure)

- ✅ Scales well (especially in managed cloud NAT services)

## ❌ Disadvantages

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

- ❌ Cannot receive unsolicited inbound connections

- ❌ Requires public IP or Elastic IP (in cloud)

- ❌ May incur cost in cloud services (e.g., AWS NAT Gateway is billed hourly + per GB)

## 📌 Summary

# Source: Chatgpt (GPT‑5 Mini) at [12/18/2025]

| Item | Value |
| --- | --- |
| NAT Gateway | Translates private IPs to a public IP for outbound internet access |
| OSI Layer | Layer 3 (Network Layer) |
| Direction | Outbound only (inbound must be initiated from inside) |
| Used In | Home routers, enterprise firewalls, cloud platforms like AWS, Azure |
| Main Benefit | Secure outbound access from private networks to the internet |