# 1-Voice calls (VoIP) Protocols

## 📞 What is Voice over IP (VoIP) Protocol?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

VoIP (Voice over Internet Protocol) is a set of communication protocols and technologies that allow voice calls to be made over the internet or any IP-based network, instead of traditional telephone lines.

🧠 Simply put: VoIP turns your voice into digital data and transmits it over the internet — like how videos or emails work.

## 🧱 Key VoIP Protocols

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Protocol | Stands For | Purpose | Layer |
| --- | --- | --- | --- |
| SIP | Session Initiation Protocol | 📞 Sets up, manages, and tears down calls | Application |
| RTP | Real-time Transport Protocol | 📤 Carries actual voice/audio data | Transport |
| RTCP | RTP Control Protocol | 📊 Monitors performance of RTP | Transport |
| H.323 | — | Legacy signaling protocol (less used) | Application |
| MGCP | Media Gateway Control Protocol | Controls media gateways in VoIP systems | Control |
| SCCP | Skinny Client Control Protocol | Cisco's proprietary protocol for VoIP | Control |

## 🔁 Common Call Flow (Simplified):

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

1. SIP: Hello, start a call between Alice and Bob

2. RTP: Stream voice data (the actual audio)

3. SIP: Call ends

## 🎯 What Each Protocol Does

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Protocol | What It Does |
| --- | --- |
| SIP | Sets up, modifies, and ends the call session |
| RTP | Transmits audio (and sometimes video) in real-time |
| RTCP | Reports latency, jitter, and packet loss |
| MGCP | Controls media gateways between VoIP & PSTN |

## 🧠 How VoIP Differs from PSTN (Landlines)

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Feature | PSTN (Traditional Phone) | VoIP (Internet) |
| --- | --- | --- |
| 📡 Transport | Analog signal over copper | Digital packets over IP network |
| 🎙️ Audio Data | Circuit-switched voice | Packet-switched audio (RTP) |
| 💰 Cost | Expensive (especially long-distance) | Cheaper or free worldwide |

## 🛠️ Real-World Examples

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Tool | Uses These Protocols |
| --- | --- |
| Skype | SIP + proprietary |
| Zoom | SIP + RTP (with enhancements) |
| Cisco Phones | SIP or SCCP |
| Asterisk PBX | SIP, RTP, MGCP |
| WhatsApp/Signal | Custom encrypted RTP + SRTP |

## 🔧 Simple VoIP Call Flow Diagram (SIP + RTP)

+------------+              +---------------+               +------------+

|  Caller    |              |   SIP Server  |               |  Receiver  |

| (Softphone)|  SIP INVITE  |  (Asterisk or |   SIP INVITE  | (Softphone)|

| or Phone   | ───────────▶ |  FreeSWITCH)  | ────────────▶ | or Phone   |

|            |              |               |               |            |

|            |  100 Trying  |               | 100 Trying    |            |

|            | ◀──────────  |               | ◀───────────  |            |

|            | 180 Ringing  |               | 180 Ringing   |            |

|            | ◀──────────  |               | ◀───────────  |            |

|            | 200 OK       |               | 200 OK        |            |

|            | ◀──────────  |               | ◀───────────  |            |

| ACK        |              |               | ACK           |            |

| ─────────▶ |              | ─────────────▶|               |            |

|            |              |               |               |            |

| RTP (Voice Media via UDP directly between both phones)               |

| ◀────────────────────────────────────────────────────────────────▶   |

|            |              |               |               |            |

| BYE        |              |               | BYE           |            |

| ─────────▶ |              | ─────────────▶|               |            |

| 200 OK     |              | 200 OK        |               |            |

| ◀───────── |              | ◀──────────── |               |            |

+------------+              +---------------+               +------------+

✅ SIP handles signaling (call setup/teardown)

✅ RTP carries the actual voice (media) between endpoints

## ✅ Summary

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Term | Description |
| --- | --- |
| VoIP | Voice communication over internet |
| SIP | Call setup, management |
| RTP | Real-time audio streaming |
| RTCP | Monitors call quality |
| H.323 | Older standard, now mostly replaced by SIP |

# 2-SIP

## 📞 What is SIP?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

SIP stands for Session Initiation Protocol.
 It’s a signaling protocol used to start, manage, and end real-time communication sessions over IP networks — like voice calls, video calls, messaging, or conferencing.

🧠 SIP doesn't carry the voice or video itself — it handles the setup and control of the session. The actual media (your voice or video) is usually sent via RTP (Real-time Transport Protocol).

## 🧱 What Does SIP Do?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Function | Example |
| --- | --- |
| 🔄 Start a call | User A dials User B — SIP sends an INVITE |
| 🔁 Manage the call | Put on hold, transfer, add video, or conference |
| ❌ End the call | User hangs up — SIP sends a BYE |

## 📦 SIP Message Flow Example

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

A ➝ B: INVITE

B ➝ A: 180 Ringing

B ➝ A: 200 OK

A ➝ B: ACK

(🔊 RTP audio begins)

A ➝ B: BYE

B ➝ A: 200 OK

## 🔧 SIP Uses These Message Types

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| SIP Method | Purpose |
| --- | --- |
| INVITE | Start a session (call) |
| ACK | Confirm a successful INVITE |
| BYE | End the call |
| CANCEL | Cancel an INVITE |
| REGISTER | Register a user/device |
| OPTIONS | Check capabilities |

## 🏢 SIP in the Real World

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Where It’s Used | How |
| --- | --- |
| 🏢 VoIP Phone Systems | Cisco, Avaya, 3CX, FreePBX |
| 💬 Messaging Platforms | Used behind the scenes for signaling |
| 📱 Apps like Skype/Zoom | Use SIP (sometimes with custom layers) |
| 📞 SIP Trunking | Businesses connect VoIP systems to the public phone network |

## 🔐 Does SIP Support Security?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

✅ Yes — with:

- SIPS (SIP over TLS): Encrypts the SIP signaling

- SRTP: Encrypts the media (audio/video) content

## 📌 Summary

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Item | Description |
| --- | --- |
| SIP | A signaling protocol for initiating, modifying, and ending real-time sessions over IP |
| Transports | Typically uses TCP, UDP, or TLS (for SIPS) |
| Used For | VoIP calls, video chat, conferencing, instant messaging |
| Media | Works with RTP (not part of SIP itself) |

# 3-SIPS

## 🔐 What is SIPS?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

SIPS stands for Secure SIP — it's SIP (Session Initiation Protocol) encrypted over TLS (Transport Layer Security).
 It ensures the signaling part of VoIP calls (like call setup, authentication, etc.) is secure and encrypted.

📞 SIP = signaling protocol for VoIP
 🔐 SIPS = SIP over TLS encryption

## 🔄 How It Works

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

- SIP usually uses UDP or TCP on port 5060 (unencrypted).

- SIPS uses TLS (typically port 5061) to encrypt the SIP signaling messages.

- It authenticates servers/clients using certificates, just like HTTPS.

❗ Note: SIPS secures the signaling — not the voice/audio media stream (RTP).
 To encrypt audio, you'd also use SRTP (Secure RTP).

## 🔍 SIP vs. SIPS vs. SRTP

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Protocol | Protects... | Encryption? | Port |
| --- | --- | --- | --- |
| SIP | Signaling | ❌ No | 5060 (UDP/TCP) |
| SIPS | Signaling | ✅ Yes (TLS) | 5061 (TCP + TLS) |
| SRTP | Audio/Video Media | ✅ Yes | RTP port range |

## 🛡️ Why Use SIPS?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Benefit | Description |
| --- | --- |
| 🔒 Data encryption | Prevents interception of call setup info |
| 🧑‍💻 User protection | Hides phone numbers, credentials, IPs |
| 🛑 Man-in-the-middle defense | Stops attackers from hijacking SIP sessions |
| ✅ Compliance | Needed for GDPR, HIPAA, or secure comms |

## 🛠️ Example SIPS URI

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

sips:alice@example.com

- Tells the SIP server/client to use TLS when sending SIP messages.

## ✅ Summary

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Item | Description |
| --- | --- |
| SIPS | Secure version of SIP (over TLS) |
| Encrypts | Call setup and signaling messages |
| Port | 5061 (TCP over TLS) |
| Does NOT encrypt | Voice/audio — use SRTP for that |
| Used in | Secure VoIP systems, enterprise calling, SIP trunks |

# 4-RTP

## 🎤 What is RTP (Real-time Transport Protocol)?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

RTP stands for Real-time Transport Protocol.
 It is a network protocol used to deliver audio and video over IP networks — such as in:

- 📞 VoIP calls

- 📹 Video conferencing

- 🖥️ Live streaming

🔊 RTP carries real-time media, like your voice or webcam, during a call — but does not provide encryption on its own.

## 🔁 RTP in Action

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

In a VoIP call:

- SIP or SIPS sets up the call.

- RTP handles the actual voice or video data (media stream).

## 📦 What RTP Does

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Feature | Purpose |
| --- | --- |
| 📤 Media Transport | Sends audio/video packets across the network |
| ⏱️ Timestamps | Keeps media playing in correct order and timing |
| 🆔 Payload Type IDs | Tells receiver what kind of media (e.g., audio codec) |
| 🔢 Sequence Numbers | Detects lost or out-of-order packets |

## 📡 RTP Packet (Simplified Structure)

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

+---------------------+

| Header (12 bytes)   |

| - Timestamp         |

| - Sequence number   |

| - SSRC ID           |

+---------------------+

| Payload (audio/video data) |

+---------------------+

## ⚠️ Is RTP Secure?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

❌ No — RTP is not encrypted by default.
 Anyone on the same network can potentially capture and listen to RTP traffic using sniffing tools like Wireshark.

✅ Use SRTP (Secure RTP) to encrypt the media stream.

## ⚙️ Protocol Stack with RTP

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Layer | Protocol |
| --- | --- |
| Application | SIP, WebRTC |
| Media Transport | RTP (or SRTP) |
| Transport Layer | UDP (mostly) |
| Network Layer | IP |

## 📱 Real-World Uses of RTP

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| App/Platform | Uses RTP? |
| --- | --- |
| Zoom | ✅ Yes |
| Skype | ✅ Yes |
| Google Meet | ✅ Yes |
| SIP VoIP Systems | ✅ Yes |
| WebRTC Browsers | ✅ Yes (SRTP required) |

## 📞 SIP + RTP VoIP Call Flow Diagram

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

This shows how SIP handles signaling (starting the call), and RTP carries the voice during the call.

Caller                    SIP Server                    Receiver

(Softphone A)             (e.g. Asterisk)               (Softphone B)

|                           |                            |

| ---- INVITE ------------> |                            |

|                           | ---- INVITE -------------> |

|                           | <--- 180 Ringing --------- |

| <--- 180 Ringing -------- |                            |

|                           | <--- 200 OK -------------- |

| <--- 200 OK ------------- |                            |

| ---- ACK ----------------> | ---- ACK ----------------> |

|                           |                            |

|       🔊 RTP Voice Stream (bi-directional)             |

| <===============================================>     |

|                           |                            |

| ---- BYE ----------------> | ---- BYE ----------------> |

| <--- 200 OK ------------- | <--- 200 OK -------------- |

## 🧾 Key Concepts:

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Phase | Protocol | Description |
| --- | --- | --- |
| Call Setup | SIP | INVITE, 180 Ringing, 200 OK, ACK |
| Media | RTP | Voice or video packets (UDP) |
| Call End | SIP | BYE and 200 OK |

## 🔐 Want to Make It Secure?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Layer | Secure Version |
| --- | --- |
| SIP | SIPS (SIP over TLS) |
| RTP | SRTP (encrypted RTP) |

✅ This ensures both your call setup and voice content are encrypted.

## ✅ Summary

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Term | Description |
| --- | --- |
| RTP | Protocol for delivering real-time media |
| Used for | Audio/video in VoIP, video chat, etc. |
| Not secure | Use SRTP to encrypt RTP traffic |
| Works with | SIP, SIPS, WebRTC, RTSP |

# 5-RTCP

## 📊 What is RTCP (Real-Time Control Protocol)?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

RTCP stands for Real-Time Control Protocol.
 It is a companion protocol to RTP (Real-Time Transport Protocol) and is used to monitor the quality and manage the delivery of audio/video streams in real-time communication.

🎧 While RTP delivers the actual media (audio/video),
 🧭 RTCP provides feedback, statistics, and control info about the stream.

## 🧩 RTP vs RTCP (Side-by-Side)

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Protocol | Role | Example |
| --- | --- | --- |
| RTP | Carries media data | Your voice/audio packets |
| RTCP | Monitors and controls RTP flow | Reports latency, jitter, packet loss |

They usually run together, with RTP on one port and RTCP on the next port number (e.g., RTP on port 5004, RTCP on 5005).

## 📦 What RTCP Does

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Function | Description |
| --- | --- |
| 📈 QoS Monitoring | Reports on packet loss, jitter, delay, and bitrate |
| 🔗 Syncing Media | Helps synchronize audio and video streams (like matching lips to sound) |
| 👥 Participant Info | Carries sender name, source ID, and contact info |
| 🚫 Session Control | Used to end sessions, manage feedback, and report issues |

## 🛠️ Common RTCP Packet Types

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Packet Type | Purpose |
| --- | --- |
| SR (Sender Report) | Sent by participants who send RTP media |
| RR (Receiver Report) | Sent by receivers to report on RTP stream quality |
| SDES (Source Description) | Provides participant info (e.g., name, email) |
| BYE | Indicates leaving the session |
| APP | For application-specific extensions |

## 🧠 Example Use Case

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

In a video call:

- Your browser sends RTP to transmit your voice.

- It also sends RTCP packets reporting:

  - Packet loss rate

  - Round-trip delay

  - Synchronization info (for lips and sound alignment)

The other side can use this info to adjust codecs or bitrates dynamically.

## 🧩 RTP + RTCP Packet Flow Diagram

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

Caller (User A)                               Receiver (User B)

|                                                |

| --- RTP (Voice/Video Packet) --------------->  |  🎤🔊

|                                                |

| <--- RTCP Receiver Report (Loss/Delay) ------  |  📈📨

|                                                |

| --- RTCP Sender Report (Sync Info) --------->  |  🧭⏱️

|                                                |

| --- RTP (More Media Packets) --------------->  |

|                                                |

| <--- RTCP BYE (End of Session) -------------   |

## 🎯 Breakdown of Key Packet Types

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| RTCP Packet Type | Who Sends It | Purpose |
| --- | --- | --- |
| SR (Sender Report) | Media sender | Timestamp, packet count, sync data (helps lip-sync video/audio) |
| RR (Receiver Report) | Media receiver | Packet loss %, jitter, delay feedback |
| SDES (Source Desc.) | Both parties | Info about sender (CNAME, name, email, etc.) |
| BYE | Leaving participant | Signals end of stream or session |

## 🔍 Example: Why RTCP Is Important

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

Imagine you're in a Zoom call:

- RTP sends your voice and video.

- RTCP monitors:

  - 🎯 Did packets arrive late?

  - 🧱 Did some audio frames get lost?

  - 📉 Should bitrate be reduced?

- Zoom’s server can dynamically adjust quality using this data.

## 💡 RTP & RTCP Port Example

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Protocol | Typical Transport | Port Example |
| --- | --- | --- |
| RTP | UDP | 5004 |
| RTCP | UDP (RTP + 1) | 5005 |

### ✅ Summary Table

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Protocol | Transmits | Security (with SRTP) | Example Use |
| --- | --- | --- | --- |
| RTP | Media (voice/video) | ✅ SRTP encrypts it | Zoom, Skype, WebRTC |
| RTCP | Control & feedback | ✅ SRTCP encrypts it | QoS monitoring |

## ✅ Summary

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Feature | Description |
| --- | --- |
| Name | Real-Time Control Protocol |
| Works with | RTP |
| Purpose | Feedback, statistics, session control |
| Port | Often RTP port + 1 |
| Used in | VoIP, video conferencing, live streaming |
| Encryption? | Yes, when combined with SRTP and SRTCP |

# 6-SRTP

## 🔐 What is SRTP?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

SRTP stands for Secure Real-time Transport Protocol.
 It is an encryption layer for RTP (Real-time Transport Protocol), used to secure the media stream in VoIP and video calls.

📞 SIP/SIPS handles call setup
 🔊 RTP/SRTP handles actual voice/audio
 🔐 SRTP ensures the voice/audio is encrypted and cannot be intercepted

## 🎯 Why Use SRTP?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

Without SRTP, VoIP audio streams sent over RTP can be intercepted using tools like Wireshark.
 With SRTP, the audio is encrypted and authenticated, preventing:

- Eavesdropping (voice spying)

- Tampering (manipulated voice)

- Replay attacks (repeating captured voice)

## 🧱 SRTP Secures…

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Component | Secured by SRTP |
| --- | --- |
| 🔊 Audio (Voice) | ✅ Yes |
| 📹 Video | ✅ Yes |
| 🪪 Identity/Auth | ✅ Yes (message authentication) |
| 📞 SIP Signaling | ❌ No — that’s SIPS (TLS for SIP) |

## 🔐 Security Features of SRTP

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Feature | Description |
| --- | --- |
| 🔒 Encryption | AES to encrypt voice/video packets |
| 🧾 Message Authentication | Ensures the packet wasn’t altered in transit |
| ⏱️ Replay Protection | Rejects old/replayed packets |

## ⚙️ How SRTP Works (Simplified)

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

- Call is established (via SIP/SIPS)

- Keys for SRTP are exchanged securely (via SDP + TLS)

- Audio/video is transmitted via RTP, but encrypted using SRTP

- Receiver decrypts and plays the media stream

## 📦 Protocol Stack

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Layer | Protocol |
| --- | --- |
| Application | SIP (or SIPS) |
| Media Signaling | SDP |
| Media Transport | SRTP (instead of RTP) |
| Transport Layer | UDP (usually) |

## 🛠️ Example VoIP Tools Supporting SRTP

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Tool | Supports SRTP? |
| --- | --- |
| Asterisk | ✅ Yes |
| FreeSWITCH | ✅ Yes |
| Cisco Phones | ✅ Yes |
| Zoom/Meet | ✅ (with extra layers) |
| WebRTC | ✅ Always uses SRTP |

## ✅ Summary

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Item | Description |
| --- | --- |
| SRTP | Secures voice/video media in VoIP |
| Encrypts | RTP streams (audio/video) |
| Uses | AES, HMAC, anti-replay |
| Works with | SIP, SIPS, WebRTC |
| Port | Same as RTP (dynamic UDP ports) |

# 7-RTSP

## 🎥 What is RTSP (Real-Time Streaming Protocol)?

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

RTSP stands for Real-Time Streaming Protocol.
 It is a network control protocol used to control streaming media servers, similar to how a remote control operates a media player.

📡 RTSP is used to start, pause, resume, and stop real-time video or audio streams over a network — without downloading the entire file.

## 🎯 RTSP Is Commonly Used For:

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

- IP cameras and CCTV systems

- Live surveillance streaming

- Streaming from media servers like VLC, Wowza, or NGINX

- Internal enterprise video systems

- Professional broadcasting workflows

## 🧪 RTSP vs RTP

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Protocol | Role |
| --- | --- |
| RTSP | Controls the stream (play, pause, stop) |
| RTP | Delivers the media data (audio/video) |

RTSP typically uses RTP under the hood to actually deliver the media content.

## 🛠️ RTSP Commands (Control Functions)

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Command | Purpose |
| --- | --- |
| OPTIONS | Ask what operations are supported |
| DESCRIBE | Get details about the stream (metadata) |
| SETUP | Set up the media transport (usually RTP) |
| PLAY | Start streaming |
| PAUSE | Pause the stream |
| TEARDOWN | Stop and disconnect the stream |

## ⚙️ Example RTSP URL

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

rtsp://username:password@192.168.1.100:554/live/stream

| Part | Meaning |
| --- | --- |
| rtsp:// | RTSP protocol scheme |
| username:password@ | (Optional) authentication |
| 192.168.1.100 | IP address of the media server |
| :554 | Default RTSP port |
| /live/stream | Path to the media stream |

## 📹 RTSP vs Other Protocols

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Protocol | Used For | Control | Media Delivery | Encryption |
| --- | --- | --- | --- | --- |
| RTSP | IP cameras, LAN streaming | ✅ Yes | RTP | ❌ Not by default |
| HLS | Web streaming (YouTube, etc.) | ❌ No | HTTP chunks | ✅ (HTTPS) |
| DASH | Adaptive video streaming | ❌ No | HTTP chunks | ✅ (HTTPS) |
| RTMP | Flash streaming/live video | ❌ No | TCP | ❌ (typically) |

## ✅ Summary

# Source: Chatgpt (GPT-5-Mini) at [1/18/2026]

| Feature | Detail |
| --- | --- |
| Name | Real-Time Streaming Protocol |
| Purpose | Control streaming audio/video in real time |
| Used with | RTP for media delivery |
| Port | Default is 554 |
| Common uses | IP cameras, VLC, media servers |
| Secure? | ❌ Not encrypted by default (can use RTSPS) |