# Datagram

## 🧱 What Is a Datagram in Network Data Structures?

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

A datagram is a self-contained, independent unit of data that is sent across a network without establishing a connection. It typically refers to Layer 3 (IP) or Layer 4 (UDP) communication.

## 🧠 Simple Definition:

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

A datagram is a complete package of data that includes all the information needed for routing and delivery — no connection or handshake is required.

## 📚 Where It Fits (OSI Model):

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Layer | Protocol | Data Unit Name |
| --- | --- | --- |
| Layer 4 | UDP | Datagram |
| Layer 3 | IP (IPv4/IPv6) | IP Datagram |
| Layer 2 | Ethernet, Wi-Fi | Frame |

## 📦 Structure of an IP Datagram

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| IP Header | Payload (e.g., UDP segment or TCP segment) |

## Example (IPv4 Datagram carrying UDP):

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Field | Example Value | Description |
| --- | --- | --- |
| Version | 4 | IPv4 |
| Header Length | 20 bytes | IP header size |
| Total Length | 48 bytes | Header + Payload |
| Source IP | 192.168.1.10 | Sender address |
| Destination IP | 8.8.8.8 | Receiver address |
| Payload | UDP Datagram (e.g. DNS request) |  |

## 🧩 Structure of a UDP Datagram (at Layer 4)

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| UDP Header | Data (App payload) |

| UDP Field | Example Value |
| --- | --- |
| Source Port | 50500 |
| Dest Port | 53 (DNS) |
| Length | 28 bytes |
| Checksum | Optional |
| Payload | DNS query |

## 🚀 Key Characteristics

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Feature | Value |
| --- | --- |
| Connectionless | No handshake required |
| Stateless | Each datagram is handled independently |
| Unreliable | No guarantee of delivery or order |
| Fast | Low overhead — good for real-time apps |
| Examples | DNS, video streaming, VoIP, gaming |

## 🧱 Datagram – Visual Example

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

Certainly! Here's a 🧱 Visual Example of a Datagram, focusing on an IP Datagram (Layer 3) carrying a UDP Datagram (Layer 4) — a classic example of connectionless transmission, such as a DNS query.

### 🧠 Example: IP Datagram with UDP Inside

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

We're sending a DNS query from your computer to a DNS server (e.g., 8.8.8.8).

### 🧱 Full Structure

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

[IP Header] + [UDP Header] + [Payload (DNS Request)]

### 🧩 IP Header (Layer 3) – 20 Bytes

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Field | Value | Description |
| --- | --- | --- |
| Version | 4 | IPv4 |
| Header Length | 20 bytes | No options |
| Total Length | 56 bytes | Header + UDP + DNS |
| Protocol | 17 | UDP |
| Source IP | 192.168.1.10 | Local device |
| Destination IP | 8.8.8.8 | DNS server (Google) |

🟥 Hex (partial):

45 00 00 38 1C 46 00 00 40 11 B1 E6 C0 A8 01 0A 08 08 08 08

### 🧩 UDP Header (Layer 4) – 8 Bytes

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Field | Value | Description |
| --- | --- | --- |
| Source Port | 55344 | Random ephemeral port |
| Destination Port | 53 | DNS |
| Length | 36 bytes | UDP + DNS data |
| Checksum | 0xA1B2 | Integrity check |

🟥 Hex:

D8 60 00 35 00 24 A1 B2

### 🟩 Payload – DNS Query (Application Data)

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Data Type | Value |
| --- | --- |
| Query | "A record for google.com" |  |

🟢 DNS Payload (partial hex):

12 34 01 00 00 01 00 00 00 00 00 00

06 67 6F 6F 67 6C 65 03 63 6F 6D 00

00 01 00 01

### 🧾 Combined Datagram (Hex)

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

45 00 00 38 1C 46 00 00 40 11 B1 E6 C0 A8 01 0A 08 08 08 08  ← IP Header

D8 60 00 35 00 24 A1 B2                                     ← UDP Header

12 34 01 00 00 01 00 00 00 00 00 00                         ← DNS Request (Payload)

06 67 6F 6F 67 6C 65 03 63 6F 6D 00 00 01 00 01

### ✅ Visual Summary

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

+--------------------+       ← IP Header

| Source IP          |

| Destination IP     |

| Protocol = UDP     |

+--------------------+

| Source Port        |       ← UDP Header

| Destination Port   |

| Length & Checksum  |

+--------------------+

| DNS Query Payload  |       ← Actual data (Application Layer)

+--------------------+

### ✅ Key Takeaways

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Element | Value |
| --- | --- |
| Datagram Type | IP Datagram with UDP inside |
| Header | IP Header + UDP Header |
| Payload | DNS Query |
| Protocol | Connectionless (no handshake) |

## 🆚 Datagram vs Segment

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Feature | Datagram (UDP/IP) | Segment (TCP) |
| --- | --- | --- |
| Protocol | UDP or IP | TCP |
| Connection | No | Yes (3-way handshake) |
| Reliability | Not guaranteed | Reliable |
| Order | Not guaranteed | Guaranteed |
| Overhead | Low | Higher |

## ✅ Summary

# Source: Chatgpt (GPT‑5 Mini) at [12/14/2025]

| Term | Datagram |
| --- | --- |
| Layer | Network (IP) or Transport (UDP) |
| Used by | UDP, IP |
| Contains | Header + payload |
| Key trait | Connectionless and fast delivery |